QA Embedded — Hardware Test Sheet / Test Log

Smart Gate Controller • Wokwi • Black-box testing

## Test Session

| Tester | Kofan Evhenii | Date | 22/09/2026 |
| --- | --- | --- | --- |
| Build / Firmware | Smart Gate Controller (Wokwi project) | Environment | Wokwi |
| Project | Smart Gate Controller | Board | Arduino Uno |
| Test Type | Functional | Result | FAIL: 4 defects found (BUG-01 – BUG-04) |

## Test Execution Rules

- Test the device as a black box.

- Record observable behavior, not assumptions about the implementation.

- For every FAIL, create a Bug Report.

- Use the same test cases for regression after firmware changes.

- Try both positive and negative scenarios.

## Test Cases

| ID | Requirement | Precondition | Action / Steps | Expected Result | Actual Result | P/F | Bug ID |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-01 | R1 | Controller is powered OFF. LIMIT_OPEN = OFF. LIMIT_CLOSE = OFF. Buttons are not pressed. | Power ON the controller and wait for startup. | State CLOSED; relays OFF | CLOSED; relays OFF; LEDs OFF; BUZZER ON | PASS | — |
| TC-02 | R2 | Controller is in CLOSED. Both relays are OFF. | Press OPEN button / send O command. | OPENING; OPEN relay ON; OPEN LED ON | OPENING; OPEN relay ON; OPEN LED ON | PASS | — |
| TC-03 | R3 | Controller is in OPENING. OPEN relay = ON. | Activate LIMIT_OPEN. | OPEN; OPEN relay OFF | Stays OPENING; OPEN relay ON; "OPEN LIMIT DETECTED" x47 | FAIL | BUG-01 |
| TC-04 | R4 | Controller is in OPEN. Both relays are OFF. | Press CLOSE button / send C command. | CLOSING; CLOSE relay ON; CLOSE LED ON | ERROR ("movement timeout"); relays OFF; ERROR LED ON; BUZZER ON | FAIL | BUG-02 |
| TC-05 | R5 | Controller is in CLOSING. CLOSE relay = ON. | Activate LIMIT_CLOSE. | CLOSED; CLOSE relay OFF | CLOSED; both relays OFF; LEDs OFF | PASS | — |
| TC-06 | R6 | Controller is in OPENING. OPEN relay = ON. | Press STOP button / send S command. | STOPPED; both relays OFF | OPENING -> STOPPED; both relays OFF | PASS | — |
| TC-07 | R6 | Controller is in CLOSING. CLOSE relay = ON. | Press STOP button / send S command. | STOPPED; both relays OFF | CLOSING -> STOPPED; both relays OFF | PASS | — |
| TC-08 | R7 | Controller is in OPENING. OPEN relay = ON. | Press OPEN again / send O again. | No unsafe/contradictory state | Stays OPENING; OPEN relay ON; repeated O ignored ("IGNORED: OPEN is not valid in current state") | PASS | — |
| TC-09 | R8 | Controller is in OPENING. OPEN relay = ON. | Press CLOSE / send C. | Safe transition; no contradictory state; never both relays ON | OPENING -> CLOSING; relays never ON simultaneously | PASS | — |
| TC-10 | R9 | Controller is operational. | Perform movement commands and observe both relay outputs during transitions. | Safe transition; never both relays ON | Relays never ON simultaneously during transitions | PASS | — |
| TC-11 | R10 | Controller is in CLOSED. LIMIT_OPEN remains inactive. | Send OPEN. Do not activate LIMIT_OPEN. Wait more than 10 seconds. | ERROR; relays OFF; ERROR LED ON; buzzer indicates error | Stays OPENING; OPEN relay ON; ERROR LED OFF; no timeout, no ERROR | FAIL | BUG-03 |
| TC-12 | R10 | Controller is in OPEN. LIMIT_CLOSE remains inactive. | Send OPEN. Do not activate LIMIT_OPEN. Send CLOSE. Do not activate LIMIT_CLOSE. Wait more than 10 seconds. | ERROR; relays OFF; ERROR LED ON; buzzer indicates error | CLOSING -> ERROR ("movement timeout"); relays OFF; ERROR LED ON; BUZZER ON | PASS | — |
| TC-13 | R11 | Controller is running. | Activate LIMIT_OPEN and LIMIT_CLOSE simultaneously. | ERROR; relays OFF; ERROR LED ON | Stays CLOSED; ERROR LED OFF; only "WARNING: both limit switches active" in log | FAIL | BUG-04 |

## Bug Reports

| Bug ID | BUG-01 |
| --- | --- |
| Requirement | R3 |
| Title | Controller stays in OPENING and OPEN relay remains ON after LIMIT_OPEN is activated |
| Severity | Critical |
| Precondition | Controller is in OPENING. OPEN relay = ON. |
| Steps to Reproduce | 1. Start simulation (initial state CLOSED). 2. Press OPEN button / send O command. Verify state = OPENING. 3. Activate LIMIT_OPEN. 4. Observe state, relays, LEDs and Serial Monitor. |
| Expected Result | STATE = OPEN; OPEN relay = OFF; CLOSE relay = OFF; OPEN LED = ON; ERROR LED = OFF |
| Actual Result | STATE = OPENING; OPEN relay = ON; CLOSE relay = OFF; OPEN LED = ON; CLOSE LED = OFF; ERROR LED = OFF; BUZZER = OFF. State does not change. |
| Reproducibility | 5/5 |
| Evidence | Serial log: "OPEN LIMIT DETECTED" repeated 47 times |
| Notes | TC-03. Motor is not stopped at the open limit. Blocks TC-04 precondition (OPEN state cannot be reached). |

| Bug ID | BUG-02 |
| --- | --- |
| Requirement | R4 |
| Title | Controller goes to ERROR ("movement timeout") instead of CLOSING when CLOSE command is sent |
| Severity | Medium |
| Precondition | Controller is in STOPPED. OPEN state cannot be reached due to BUG-01. |
| Steps to Reproduce | 1. Start simulation (initial state CLOSED). 2. Send O command, state becomes OPENING. 3. Send S command, state becomes STOPPED. 4. Send C command, state becomes CLOSING. 5. Wait and observe state, relays, LEDs and Serial Monitor. |
| Expected Result | STATE = CLOSING; CLOSE relay = ON; OPEN relay = OFF; CLOSE LED = ON; OPEN LED = OFF; ERROR LED = OFF |
| Actual Result | STATE = ERROR; OPEN relay = OFF; CLOSE relay = OFF; OPEN LED = OFF; CLOSE LED = OFF; ERROR LED = ON; BUZZER = ON |
| Reproducibility | 5/5 |
| Evidence | Serial log: Initial state: CLOSED COMMAND: O STATE: CLOSED -> OPENING COMMAND: S STATE: OPENING -> STOPPED COMMAND: C STATE: STOPPED -> CLOSING ERROR: movement timeout STATE: CLOSING -> ERROR |
| Notes | TC-04. Related to BUG-01. Verify the time between "STOPPED -> CLOSING" and "ERROR": if it is 10 s or more without LIMIT_CLOSE, this matches R10 (see TC-12) and the bug may be rejected. |

| Bug ID | BUG-03 |
| --- | --- |
| Requirement | R10 |
| Title | No movement timeout during opening: controller stays in OPENING with OPEN relay ON when LIMIT_OPEN is not reached |
| Severity | High |
| Precondition | Controller is in CLOSED. LIMIT_OPEN remains inactive. |
| Steps to Reproduce | 1. Start simulation (initial state CLOSED). 2. Send O command. 3. Do not activate LIMIT_OPEN. 4. Wait more than 10 seconds. 5. Observe state, relays, LEDs, buzzer and Serial Monitor. |
| Expected Result | STATE = ERROR; OPEN relay = OFF; CLOSE relay = OFF; ERROR LED = ON; buzzer indicates error |
| Actual Result | STATE = OPENING (no transition to ERROR); OPEN relay = ON; CLOSE relay = OFF; OPEN LED = ON; ERROR LED = OFF; BUZZER = OFF |
| Reproducibility | 5/5 |
| Evidence | Serial log: Initial state: CLOSED COMMAND: O STATE: CLOSED -> OPENING (no timeout message) |
| Notes | TC-11. Closing timeout works correctly (TC-12), opening timeout does not. |

| Bug ID | BUG-04 |
| --- | --- |
| Requirement | R11 |
| Title | Controller does not enter ERROR when LIMIT_OPEN and LIMIT_CLOSE are active simultaneously (only WARNING in log) |
| Severity | High |
| Precondition | Controller is running (initial state CLOSED). |
| Steps to Reproduce | 1. Start simulation. 2. Activate LIMIT_OPEN and LIMIT_CLOSE simultaneously. 3. Observe state, relays, LEDs, buzzer and Serial Monitor. |
| Expected Result | STATE = ERROR; OPEN relay = OFF; CLOSE relay = OFF; ERROR LED = ON |
| Actual Result | STATE = CLOSED; OPEN relay = OFF; CLOSE relay = OFF; OPEN LED = OFF; CLOSE LED = OFF; ERROR LED = OFF; BUZZER = OFF |
| Reproducibility | 5/5 |
| Evidence | Serial log: "WARNING: both limit switches active" repeated (4+ times) |
| Notes | TC-13. Sensor fault is detected but not handled: controller remains operational. |

## Session Summary

| Total tests | 13 | Passed | 9 |
| --- | --- | --- | --- |
| Failed | 4 (TC-03, TC-04, TC-11, TC-13) | Blocked | 0 |
| Bugs reported | 4 (BUG-01 – BUG-04) | Critical/High | 3 (BUG-01 Critical; BUG-03, BUG-04 High) |
| Coverage notes | R1–R11 covered by TC-01 – TC-13. Defects in open limit handling (R3), closing from OPEN (R4, precondition not reachable), opening timeout (R10) and simultaneous limits (R11). Closing timeout works (TC-12). Regression of TC-03, TC-04, TC-11, TC-13 required after fix. | Build result | FAIL — build not accepted (1 Critical, 2 High defects) |